# ELECTENG 311 — Smart Power Bank

# Independent OVP / OCP / OTP Protection Subsystem

Derivation of Circuit Parameters, Steady-State Operating Conditions and Control Parameters

# 1. Design Basis and Assumptions

This subsystem detects overvoltage, overcurrent and overtemperature faults and disconnects the LIC storage bank from the boost converter entirely in hardware, independent of the ATmega328PB. It sits electrically ahead of the isolation switch and is powered directly from VSTOR, so it remains functional even if the microcontroller has crashed or has not yet been programmed.

| Parameter | Value | Source |
| --- | --- | --- |
| VSTOR range | 2.5 V – 3.8 V | Slide |
| VOUT | 5.0 V | Boost converter |
| Output power range | 0.5 W – 5 W | Slides |
| Reference voltage VREF | 2.5 V ± 0.5 % | LM4040C25FTA datasheet |

# 2. Reference Generator — LM4040C25FTA

The LM4040 is a shunt reference: it only regulates while current is actively pushed through it from VSTOR. Rbias sets that current.

Ibias = (VSTOR − VREF) / Rbias

Design target: keep Ibias well inside the device's rated operating-current window across the full VSTOR range, while keeping bias power low.

Choosing Rbias = 1 kΩ, sized at the mid-range point VSTOR = 3.15 V

Ibias(3.15 V) = (3.15 − 2.5) / 1000 = 0.65 mA

## 2.1 Steady-state current across the VSTOR range

| VSTOR | Ibias | Note |
| --- | --- | --- |
| 3.8 V (full) | 1.30 mA | Within typical device rating |
| 3.15 V (50% SOC) | 0.65 mA | Design point |
| 2.7 V (near-empty) | 0.20 mA | Reduced margin |
| 2.5 V (rated minimum) | 0.00 mA | Reference has zero bias current — cannot regulate |

# 3. Overvoltage Protection (OVP)

Trip target: 10% margin above the 5 V nominal output, to avoid nuisance tripping on normal ripple while catching genuine faults promptly.

VOUT,trip = 5.0 V × 1.10 = 5.5 V

## 3.1 Sense divider derivation

The divider must present exactly VREF (2.5 V) to the comparator's plus input at the moment VOUT crosses 5.5 V:

Vsense = VOUT × R2 / (R1 + R2) = VREF  at VOUT = VOUT,trip

R2 / (R1 + R2) = 2.5 / 5.5 = 0.4545

Choosing R2 = 10 kΩ (standard value):

R1 = R2 × (1/0.4545 − 1) = 10 kΩ × 1.2 = 12 kΩ

## 3.2 Steady-state operating point

| Condition | VOUT | Vsense | Comparator state |
| --- | --- | --- | --- |
| Normal | 5.0 V | 2.27 V | LOW (Vsense < VREF) |
| Trip threshold | 5.5 V | 2.50 V | Transition point |
| Fault example | 5.7 V | 2.59 V | HIGH (Vsense > VREF) |

Divider loading at trip: Idiv = VOUT,trip / (R1+R2) = 5.5 V / 22 kΩ ≈ 250 µA, negligible against load currents in the ampere range.

# 4. Overcurrent Protection (OCP)

## 4.1 Worst-case storage-side current

Imax = Poutmax / (Vout) = 5 W / 5V ≈ 1 A

## 4.2 Trip current

Applying 30% margin over the worst-case continuous current, consistent with common OCP design practice:

Itrip = 1 A × 1.50 ≈ 3.0 A

## 4.3 Shunt selection

From the current-sense resistor options in the BOM, choosing Rshunt = 0.047 Ω:

Vshunt(Itrip) = 1.5 A × 0.047 Ω = 75 mV

Pshunt(Imax) = Imax² × Rshunt = 1² × 0.047 = 0.047  W  (continuous, worst case)

# 5. Digital Logic — OR Stage and NAND Latch

## 6.1 Logic supply and voltage levels

The 74HC32, 74HC00 and both TLV9162 comparators share the VSTOR rail. As rail-to-rail CMOS/near-rail devices, their HIGH output level tracks VSTOR directly rather than a fixed logic supply:

| VSTOR | Logic HIGH (approx.) | 74HC input HIGH threshold (≈0.7×VSTOR) |
| --- | --- | --- |
| 3.8 V | 3.6 – 3.7 V | 2.66 V |
| 3.3 V | 3.1 – 3.2 V | 2.31 V |
| 2.7 V (practical floor) | 2.5 – 2.6 V | 1.89 V |

Because the driving and receiving gates share the same supply, the HIGH level always clears the receiving threshold with margin, at any point in the VSTOR range — this is a benefit of single-rail design that would not hold if the logic and comparators used mismatched supplies.

## 6.2 3-input OR — logic table

| OVP_out | OCP_out | OTP_out | FAULT |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 |
| 0 | 1 | 0 | 1 |
| 0 | 0 | 1 | 1 |
| 1 | 1 | 1 | 1 |

## 6.3 NAND latch — derivation of stable and forced states

Gate 2 output Q and Gate 3 output Q_bar are each NAND functions of one external input and the other gate's current output:

Q = NOT(FAULT_bar AND Q_bar)

Q_bar = NOT(RESET_bar AND Q)

| FAULT_bar | RESET_bar | State | Q | Q_bar |
| --- | --- | --- | --- | --- |
| 0 | 1 | SET (fault forced) | 1 | 0 |
| 1 | 0 | RESET (cleared) | 0 | 1 |
| 1 | 1 | HOLD (memory) | previous | previous |
| 0 | 0 | Invalid — avoid | 1 | 1 (breaks Q/Q_bar complementarity) |

# 6. Component Price List

| Component | Component | Price (NZD) |
| --- | --- | --- |
| Reference Gen x 1 | LM4040C25FTA | $1.12 |
| NTC x 1 | B57164K0103J000 | $0.94 |
| MCU x 1 | ATMEGA328PB-AU | $2.99 |
| OR gates x 1 | 74HC32D,653 | $0.40 |
| NAND Gates x 2 | 74HC00D,653 | $0.80 |
| ISO Switch x 1 | TPS22917DBVR | $0.95 |
| High side current sensor | ZXCT1009FTA | $2.22 |