Group 16 project proposal

Derrick Shen, Jimmy Li, Kiran Elango, Leon Fang, Ryder Hutton, Sanuga Dheerasinghe

30/7/2026

1. Introduction

The following project proposal details the high-level design of a smart power bank. This is split into three subsystems: Charger, Boost converter, and Smart systems.

2. Design

Block diagram of the entire system:

Subsystem overview and requirements:

Charger

Charger specifications:

Input voltage: 5V DC, output voltage range: 0 – 3.5V DC, voltage regulation (ripple): < 5%.

Max output current: 0.5A DC.

PCB: 2-layer PTH with SMT and TH devices, smaller than 100mm x 100mm.

Protection: Isolation switch protects battery against overcurrent.

Efficiency: > 65%, Transient time: < 15ms.

Simulation results in PLECS:

Output voltage: 3.35V DC, minimal voltage ripple, output current: 0.479A, efficiency: 66.5%, transient time: 10ms.

Design calculations:

V+ = 3.5 * (8200) / (3300+8200) = 2.496V.

For BC557B PNP BJT: VBE(on) = -0.62V (typical), I(max) = 0.62 / 1.2 = 517mA, however in simulations VBE(on) = -0.575V justifying Rsense = 1.2Ω.

Max output voltage from LM358 OpAmp is 5 – 1.5 = 3.5V.

VGS(th) = -2V(min), -4V(max), enables PMOS to be turned off for current limiting.

Boost converter

3. Team composition

For our group of 6, it is proposed that two members will work on each of the 3 subsystems with no integration expected.

| Boost converter | Charger | Smart systems |
| --- | --- | --- |
| Jimmy Li | Derrick Shen | Ryder Hutton |
| Leon Fang | Kiran Elango | Sanuga Dheerasinghe |

Smart Systems:

The project uses a capacitive touch sensor as a user input for the power bank. When a finger touches the sensing pad, it changes the pad’s capacitance. The sensing circuit detects this change and sends a digital signal to the ATmega328PB. The MCU could then possibly use this input to turn the output on or off, display the battery status, or activate another user-interface function. The exact purpose of the touch input is not specified in the provided architecture. A capacitive sensor is suitable because it has no moving parts and can operate through a thin plastic enclosure.

Bibliography:

BC557B datasheet: https://www.onsemi.com/download/data-sheet/pdf/bc556b-d.pdf

LM358 datasheet: https://www.ti.com/lit/ds/symlink/lm358.pdf

IRF4905 datasheet:  https://www.infineon.com/assets/row/public/documents/24/49/infineon-irf4905-datasheet-en.pdf

https://www.ti.com/lit/an/slyt674/slyt674.pdf